DD/MM/YYYY

Proposed C300

SCHEDULE 3 TO CLAUSE 32.07 RESIDENTIAL GROWTH ZONE

Shown on the planning scheme map as RGZ3

COASTAL INCREASED HOUSING DIVERSITY AREAS

1.0	Requirements of Clause 54 and Clause 55

| DD/MM/YYYY Proposed C300 | Standard | Requirement |
| --- | --- | --- |
| Minimum street setback | A3 and B6 | This requirement does not apply to sites in a heritage overlay. The average distance of the setbacks of the front walls of the existing buildings on the abutting allotments facing the front street or 5.5 metres, whichever is the lesser. |
| Site coverage | A5 and B8 | Maximum site coverage 70% |
| Permeability | A6 and B9 | None specified |
| Landscaping | B13 | None specified |
| Side and rear setbacks | A10 and B17 | None specified |
| Walls on boundaries | A11 and B18 | None specified |
| Private open space | A17 | At least one part of the private open space should consist of secluded private open space with a minimum area of 20 square metres and a minimum dimension of 3 metres at the side or rear of the dwelling with convenient access from a living room. |
|  | B28 | A dwelling or residential building should have private open space consisting of: - An area of 20 square metres of secluded private open space at the side or rear of the dwelling or residential building with a minimum dimension of 3 metres and convenient access from a living room; or - A balcony of 8 square metres with a minimum width of 1.6 metres and convenient access from a living room; or - A roof-top area of 10 square metres with a minimum width of 2 metres and convenient access from a living room. |
| Front fence height | A20 and B32 | None specified |

DD/MM/YYYY

Proposed C300

2.0	Maximum building height requirement for a dwelling or residential building

A building used as a dwelling or a residential building must not exceed a height of 10.5 metres

DD/MM/YYYY

Proposed C300

3.0	Application requirements

None specified.

DD/MM/YYYY

Proposd C300

4.0	Decision guidelines

The following decision guidelines apply to an application for a permit under clause 32.07, in addition to those specified in clause 32.07 and elsewhere in the scheme:

Buildings and works & dwellings

- The requirements of Clause 22.63 in relation to the siting, height, scale, materials and form of proposed buildings.