Schedule 2 to the General Residential Zone.

| Requirements | GRZ1 Schedule 1 | GRZ2 Schedule 2 – Incremental Change Areas |
| --- | --- | --- |
| Height | Should not exceed a height of 9 metres, unless the slope of the natural ground level at any cross section wider than 8 metres of the site of the building is 2.5 degrees or more, in which case the maximum building height should not exceed 10 metres. | Must not exceed a height of 9 metres, unless the slope of the natural ground level at any cross section wider than 8 metres of the site of the building is 2.5 degrees or more, in which case the maximum building height must not exceed 10 metres. |
| Landscaping | None specified. | One canopy tree per dwelling. |
| Private open space – single dwelling | A dwelling should have private open space consisting of an area of 80 square metres or 20 per cent of the area of the lot, whichever is the lesser, but not less than 40 square metres. At least one part of the private open space should consist of secluded private open space with a minimum area of 25 square metres and a minimum dimension of 3 metres at the side or rear of the dwelling with convenient access from a living room. | An area of 60 square metres, with one part of the private open space to consist of secluded private open space at the side or rear of the dwelling or residential building with minimum area of 40 square metres, a minimum dimension of 5 metres and convenient access from a living room. It cannot include a balcony or roof top terrace. |
| Private open space – two or more dwellings | A dwelling or residential building should have private open space consisting of: - An area of 40 square metres, with one part of the private open space to consist of secluded private open space at the side or rear of the dwelling or residential building with a minimum area of 25 square metres, a minimum dimension of 3 metres and convenient access from a living room; or - A balcony of 8 square metres with a minimum width of 1.6 metres and convenient access from a living room; or - A rooftop area of 10 square metres with a minimum width of 2 metres and convenient access from a living room. | An area of 60 square metres, with one part of the private open space to consist of secluded private open space at the side or rear of the dwelling or residential building with minimum area of 40 square metres, a minimum dimension of 5 metres and convenient access from a living room. It cannot include a balcony or roof top terrace. |
| Application requirements | None specified. | Subdivision: When any of the lots being created are less than 500 square metres, a subdivision application must be accompanied by a development application or approved planning permit plans for the site for the construction of the dwellings. |

In practice, the changes mean that any new development will be more consistent with the garden character of the area, by requiring a greater area of private open space than is currently the case in GRZ1 for two or more dwellings and by requiring a canopy tree for every new dwelling.