--/--/2016

C325

SCHEDULE 39 TO THE DESIGN AND DEVELOPMENT OVERLAY

Shown on the planning scheme map as DDO39

ST LEONARDS GROWTH AREA 2

--/--/2016

C325

1.0	Design objectives

General

To facilitate the orderly and integrated residential development of the area.

To promote a mix of lot sizes including higher density lots adjacent to open space.

Access

To improve vehicle and pedestrian access to and from Murradoc Road and to established residential areas.

To provide a central connector road between Leviens Road and Bluff Road.

To set the location of future vehicle connections to the west.

Open Space

To ensure local parks are conveniently connected to linear open space and/or the connector road network.

To provide an off-road linear open space network and pathway around the growth area.

To encourage a subdivision design that avoids and minimises the loss of remnant vegetation both on-site and in abutting roadside reserves.

To establish Ibbotson Street as a high amenity, high conservation value open space trail.

To manage the interface between linear and buffer open space, remnant vegetation and residential development.

To ensure best practice stormwater management objectives (quantity, quality. volume) are met to prevent adverse impact on downstream areas.

--/--/2016

C325

2.0	Buildings and works

Permit requirements

A permit is not required to construct a building or construct or carry out works.

--/--/2016

C325

3.0	Subdivision

Design requirements

Subdivision of land should  meet the following requirements where relevant:

General

Subdivision should not result in the further fragmentation of land where it would prevent orderly development in accordance with the objectives and requirements of this schedule.

Subdivision sequencing should generally support the efficient delivery of service infrastructure.

Subdivision should be generally in accordance with the St Leonards Growth Area 2 Outline Development Plan 2015 that forms part of this schedule.

Subdivision should implement the requirements of the St Leonards Growth Area 2 Shared Infrastructure Funding Plan as appropriate.

Urban Design

Subdivision should provide a range of lot sizes and densities to encourage a variety of housing types.

Open space and drainage reserves must be interfaced by roads and/or dwelling frontages on at least three sides.

Subdivision should provide for the retention, where appropriate, of high arboricultural value scattered remnant River Red Gum, Drooping Sheoak and vegetation patches in future road reserves or open space reserves.

Ibbotson Street

Subdivision should provide for the discontinuation of Ibbotson Street, between Murradoc Road and Bluff Road, and the establishment of this land as a greenway for the retention and protection of vegetation and accommodation of pedestrian and cyclist traffic.

Subdivision must not provide direct vehicle or road access to the greenway.

Existing vehicle access to Ibbotson Street from abutting properties shall remain until redirected into the new internal road network or Murradoc Road.

Subdivision should provide for the inclusion of existing vegetation immediately adjacent to Ibbotson Street (i.e. on the boundary) into the greenway.

Subdivision should ensure residential lots have frontage to the greenway.

Subdivison should provide an open space buffer of 10 metres (generally) between Ibbotson Street and residential development. The design of the buffer should give consideration to bushfire risk, fencing treatments, vegetation (existing and proposed) and pedestrian/bicycle access.

Stormwater Management

Subdivision must set aside land for drainage purposes to meet peak discharge limits and water sensitive urban design elements in accordance with performance objectives of Urban Stormwater: Best Practice Environmental Management Guidelines (CSIRO, 1999), Infrastructure Design Manual, and CoGG Design Notes.

The design of retarding basins must have sufficient land area set aside for heavy vehicle access and sediment drying; as well as measures to mitigate mosquito breeding in accordance with best practice guidelines.

Easement creation, widening and/or realignment as necessary to ensure adequate provision for pipe-laying and maintenance, and identify overland flow paths, both within the development area, and to external affected land.

A stormwater management system must be designed to ensure that:

- peak discharge rates and pollutant loads of all stormwater leaving the site post development are regulated to integrate with downstream infrastructure, at no greater than pre-development rates.

- no adverse impacts to any surrounding land, upstream or downstream.

- ecological impacts of increased overall volume of freshwater inputs into the saline environment of Swan Bay (Ramsar wetlands site) have been specifically addressed.

Traffic and Pedestrian Movements

Subdivision should provide a movement network that:

- promotes a high degree of internal permeability for a variety of transport modes, including the provision of a centrally located north-south connector road.

- enables integration with St Leonards, by utilising the surrounding road network, including a vehicle link to Pearl Bay Passage and a pedestrian link to Diver Dan Lane.

- identifies necessary upgrades of abutting roads and road intersections in the vicinity.

- sets the location of possible future road access to the west.

- provides an integrated and continuous network of safe and convenient footpaths, shared paths and bicycle lanes including connections to the greenway.

- improves connectivity to St Leonards, particularly Murradoc Road via a new footpath on McBeth Street.

Public Open Space

Subdivision should provide an open space contribution (in cash or land or a combination of both) to a minimum of 10% of the developable residential land generally as specified in the St Leonards Growth Area 2 Shared Infrastructure Funding Plan. Encumbered land must not be credited as Public Open Space including land required for the future drainage basins.

Subdivision should provide unencumbered open space (to form part of the open space contribution) of the following general widths:

- 10 metres for Buffer Open Space;

- 12 metres for Linear Open Space (Leviens Road);

- 10 metres for Linear Open Space and 5 metres for Linear Open Space abutting stormwater retarding basins (Bluff Road).

Footpaths (and possibly services) should not be provided in the Leviens and Bluff Road Reserves and instead constructed on parallel abutting linear open space within the growth area, subject to arboriculture assessments. This footpath (i.e. shared 2.5 metre wide path) should ‘loop’ around the site.

Subdivision should provide for two 1 hectare parks located generally as shown in the St Leonards Growth Area 2 Outline Development Plan 2015.

Application requirements

An application must be accompanied by the following information:

Stormwater Management

A Stormwater Management Plan responding to flooding, stormwater and drainage conditions generally in accordance with the principles outlined in  the Afflux Consulting Report dated September 2014.

Traffic and Pedestrian Movements

A Road Network and Traffic Management Plan generally in accordance with the recommendations of the Cardno Report dated November 2014.

Public Open Space

Conceptual plans for all areas of public open space, including the Ibbotson Street greenway, showing general layout and indicative landscape treatments (such as seating, play spaces and paving materials) with any infrastructure being in accordance with the standards set out in Council’s Sustainable Communities Infrastructure Development Guidelines May 2010 and the use of local indigenous plant species where appropriate.

An arboricultural assessment of the health, retention value and recommended protective measures for all trees within the growth area and within Bluff Road, Leviens Road and the Ibbotson Street greenway.

Vegetation management plans for Leviens Road and Bluff Road showing how impacts on existing vegetation from new intersecting roads, road pavement widening and service installation can be avoided or minimised.

A Weed Management and Eradication Plan.

An Environmental Assessment that includes:

- Assessment of the land by a suitably qualified environmental professional detailing the level and location of any soil contamination. If the responsible authority is satisfied that significant levels of contamination have been found:

- A certificate of environmental audit must be issued for the land in accordance with Part IXD of the Environment Protection Act 1970; or

- An environmental auditor appointed under the Environment Protection Act 1970 must make a statement in accordance with Part IXD of the Environment Protection Act 1970 that the environmental conditions of the land are suitable for the sensitive use.

Exemption from notice and review

An application to subdivide land is exempt from the notice requirements of Section 52(1)(a), (b) and (d), the decision requirements of section 64(1), (2) and (3) and the review rights of section 82(1) of the Act.

--/--/2016

C325

4.0	Decision guidelines

Before deciding on an application the Responsible Authority must consider:

Whether the subdivision design is generally in accordance with the St Leonards Growth Area 2 Outline Development Plan 2015.

Whether the subdivision is consistent with, and implements the requirements of, the St Leonards Growth Area 2 Shared Infrastructure Funding Plan.

St Leonards Growth Area 2 Outline Development Plan 2015