Great Western Hotel Heritage Place

177 Aberdeen Street,

Newtown

INCORPORATED PLAN

(Draft)

October 2016

This document is an incorporated document in the Greater Geelong Planning Scheme pursuant to section 6(2)(j) of the Planning and Environment Act 1987.

1.0	Application

This plan is incorporated into the Greater Geelong Planning Scheme under the Schedule to Clause 81.01.  It applies to the Great Western Hotel, 177 Aberdeen Street, Newtown, identified in the Schedule to the Heritage Overlay of the Greater Geelong Planning Scheme as HO1990.

No permit required

Pursuant to Clause 43.01-2, for land in HO1990, a permit is not required to:

2.1	Remove, alter or construct driveways, pathways, car parks, and kerbs and channels with the site.

2.2	Demolish or alter single storey building fabric not identified in the statement of significance for the Great Western Hotel in the Newtown West Heritage Review (2016) so long as:

    - There is no physical change to the significant two storey hotel building fabric.

    - The alterations are constructed of non-highly reflective materials.

2.3	Construct a building (including additions to the significant two storey hotel) provided that:

    - There is no physical impact, intrusion or change to the significant two storey hotel building fabric.

    - The proposed wall and roof heights are no higher than the significant two storey hotel walls and roofs.

    - Any two storey addition is set back from the Aberdeen Street and Shannon Avenue significant two storey facades a distance equivalent to the significant two storey return parapets in the north-west corner (fronting Aberdeen Street) and south-east corner (fronting Shannon Avenue).

    - The additions are constructed of non-highly reflective materials.

  - Install and display a sign on non-significant fabric only (not connected to, situated on or projecting over the significant two storey hotel building).

  - Install plant or equipment associated with hot water services or central heating units provided that the equipment is not attached to the principal, significant north and east hotel facades and significant north, south and east two storey hipped roof faces.

  - Construct roof-mounted solar panels provided that the equipment is not situated on those parts of the significant two storey hotel roofs that face directly towards Aberdeen Street and Shannon Avenue.

  - Construct boundary fencing and internal fencing so long as the fencing is non-reflective.