--/--/----

Proposed C433ggee

# SCHEDULE 57 TO CLAUSE 43.02 DESIGN AND DEVELOPMENT OVERLAY

Shown on the planning scheme map as DDO57.

PAKINGTON NORTH PRECINCT

# 1.0

--/--/----

Proposed C433ggee

# Design objectives

To support opportunities for urban renewal, commercial development, and increased housing density by implementing the Pakington North Urban Design Framework (City of Greater Geelong, May 2024).

To create a vibrant mixed-use precinct with retail, offices and community spaces at lower levels and residential spaces above.

To promote high quality medium and higher-density built form which transitions to sensitive interfaces including heritage buildings and adjoining established low scale residential areas and protects the amenity of existing and future residents.

To establish a vibrant public realm which reinforces the sense of place, supports multi modal movement, and improves local accessibility and amenity.

To protect the amenity of key public spaces including footpaths, future plazas and new public open spaces from overshadowing impacts.

# 2.0

--/--/----

Proposed C433ggee

# Buildings and works

The following buildings and works requirements apply to an application to construct a building or construct or carry out works.

A permit may be granted to vary a discretionary requirement expressed with the term ‘should’.

A permit cannot be granted to vary a requirement expressed with the term ‘must’.

A permit cannot be amended (unless the amendment does not increase the extent of non-compliance) for buildings and works that do not meet a requirement expressed with the term ‘must’.

Building and floor heights

Development should not exceed the preferred maximum building heights specified in Map 1 to this schedule.

Provided that the below criteria are met, the preferred maximum building height does not include architectural features, masts and building services. Roof top mechanical equipment such as plant rooms, lift overruns, solar collectors and other such equipment should be sited so achieve all of

the following:

Not more than 50% of the roof area is occupied by equipment (other than solar panel or greening);

The equipment is set back on all sides, no less than 3 metres from the edge of the building, or otherwise located to minimise additional overshadowing and reduce visual impact;

The equipment does not exceed the height limit by more than 3.6 metres; and The equipment and screening is integrated into the design of the building;

to the satisfaction of the Responsibly Authority.

Buildings should provide the floor-to-floor heights set out in Table 1.

Table 1: Floor-to-floor heights

Floor

Use

Minimum floor-to-floor

dimension

Ground

All

4.0 metres

Level 1 and above

Residential

3.2 metres

Floor

Use

Minimum floor-to-floor

dimension

Non-residential (including car parking)

3.5 metres

Street interface

Development should not exceed the preferred maximum street wall heights specified in Table 2.

Development must meet the mandatory ground level setbacks specified in Table 2 and Map 1.

Development should  meet the other street interface requirements specified in Table 2 and Map 1.

Development should retain heritage frontages as indicated in Map 1 to this schedule.

Table 2: Street interface

[Editing Note: Mandatory minimum ground level setback to: Pakington Street, Church Street*, Gordon Avenue.]

[Editing Note: Preferred minimum ground level setback to all other streets (plus 2-8 Church Street).]

| Location (see Map 1 to this schedule) | Mandatory minimum ground level setback | Preferred maximum street wall/podium height | Preferred minimum setback above street wall/podium |
| --- | --- | --- | --- |
| Type 1A | 0 metres | 8 metres | 5 metres |
| 28-96 and 67-103 Pakington Street |  |  |  |
| Note: only applies to portion of 28-64 Pakington Street south of alignment with Britania Street) |  |  |  |
| Type 1B | 0 metres | 11 metres | 5 metres |
| 53-57 and 63 Pakington Street |  |  |  |
| Type 1C | 0 metres | 15 metres | 5 metres |
| 21-29 Church Street |  |  |  |
| 9-27, 31-49, 8-20, 24-64 Pakington Street |  |  |  |
| Note: only applies to portion of 28-64 Pakington Street north of alignment with Britania Street) |  |  |  |
| Type 1D | 3 metres | None specified | None specified |
| 2-8 Church Street 10-28 and 17-19 Church Street |  |  |  |
| 2-6 and 1-7 Pakington Street |  |  |  |
| Type 2A | 3 metres | 15 metres | 5 metres |
| Type 2B | 2 metres | 8 metres | 2 metres |

| Location (see Map 1 to this schedule) | Mandatory minimum ground level setback | Preferred maximum street wall/podium height | Preferred setback above street wall/podium |
| --- | --- | --- | --- |
| All properties abutting Collins Street, Maitland Street, Anglesea Terrace, Waterloo Street, Clonard Avenue (south) |  |  |  |
| Type 2B All properties abutting Clonard Avenue (north), Brittania Street (south), Isabella Street | 2 metres | 11 metres | 2 metres |
| Type 2B All properties abutting Britannia Street (north) Isabella Street | 2 metres | 15 metres | 2 metres |
| Type 3 | 0 metres | 15 metres | 3 metres |
| New streets |  |  |  |

Residential interfaces

Development adjoining sites in the General Residential Zone and Neighbourhood Residential Zone should be in accordance with Figures 1-4 and Map 2 to this schedule.

Figure 1. Type 6A residential interface

Figure 2. Type 6B residential interface

Figure 3. Type 6C residential interface

Figure 4. Type 6D residential interface

Laneway interface

Development abutting laneways should be in accordance with Figures 5-7 and Map 2 to this schedule.

Figure 5. Type 4A laneway interface

Figure 6. Type 4B laneway interface

Figure 7. Type 4C rear interface

Open space interface

New open spaces and/or plazas should be provided generally in the locations indicated on Map 1 to this schedule. These spaces should be publicly accessible, open to the sky and receive good

solar access.

Development abutting the north side of new open spaces should be in accordance with Figure 8 and Map 1 to this schedule.

Development should not overshadow more than 50% of new open spaces and / or plazas between 10am and 2pm on 22 September.

Figure 8. Open space interface

[Editing Note: Delete diagram above and replace with diagram below showing label spring equinox and 10am-2pm.

[Editing Note: Edit diagram below to refine building envelope to be more generic, label as max 50% and edit spring equinox line to correct angle.

Building separation

Balconies should not encroach into side setbacks.

Where a site abuts another site subject to this schedule the minimum side setbacks at Figures 9 and 10 apply.

Figure 9. Preferred side separation for buildings built to boundary

Figure 10. Preferred side separation for buildings not built to boundary

Building design

Encourage developments at corner sites that address both street interfaces.

Encourage the use of natural, tactile, and visually interesting materials and façade articulation

Create visual interest in upper levels t and ensure passive surveillance of the public realm.

Incorporate an interim façade strategy when blank walls are visually prominent while adjoining properties are being realised.

Discourage development that mimics or replicates adjacent heritage styles.

Encourage integrated  screening that manages on and off site amenity outcomes.

Incorporate façade design and lighting that enhances safety and security after hours including along main streets, side streets and laneways.

Ensure improved pedestrian amenity and permeability across the Pakington Strand Strategic Site.

Minimise the potential for off-site impacts, including from noise, fumes, odour or vibrations through the use of setbacks, acoustic barriers, high performance glazing, internal layout, or other relevant measures.

Ensure passive surveillance for developments that directly abut or are adjacent to public open spaces with the provision of clear sight lines, views and activation to these spaces.

Integrate landscaping with building design by incorporating green walls, green roofs, roof top gardens, landscaped balconies and functional courtyards to enhance biodiversity and visual amenity.

Encourage the provision of functional open spaces (e.g. central courtyards) and landscaped areas with deep soil planting to support canopy trees around buildings.

Active frontages

Encourage the use of features which complement the existing character of Pakington Street, including the Heritage Core to the south, such as plinths, depth of façade articulation, and tactile materials.

Encourage visual and physical permeability between the building and the street by incorporating measures such as doors facing the street, operable windows and direct residential access for ground level residences.

Design buildings to address the street, with legible and direct entries to support street activation whilst clearly distinguishing residential and commercial entrances.

Allow a maximum 75% glazing on any façade and provide 50% visually permeable balcony balustrades.

Solar access, wind and weather

Incorporate weather protection such as awnings, openings and architectural detail that are well detailed and attractive when viewed from the street.

Ensure weather protection measures do not impinge existing or future street trees.

Address wind gust management in building design, without the need for additional protective screens, incidental add-ons and landscaping in public spaces.

Limit overshadowing impacts on the existing secluded private open space of adjacent residential properties outside the precinct.

Development should ensure solar access as specified in Table 3.

Table 3: Solar access

Location	Solar access requirement

Pakington Street

Maintain solar access to the opposite footpath on Pakington Street, measured 4 metres from the property boundary, between 10am and 2pm on 22 September.

Waratah Street

Maintain solar access to the southern footpath on Waratah Street, between 10am and 2pm on 22

September.

All other streets

Maintain solar access to southern footpaths where possible.

Public open space (including parks and plazas)

Maintain a minimum 50% solar access between

10am and 2pm on 22

September.

Buildings and works with an overall height equal to, or greater than 16 metres:

- Must not cause unsafe wind conditions as specified in Table 4 on public land, publicly accessible areas on private land, private open space and communal open space; and

- Should achieve comfortable wind conditions as specified in Table 4 on public land, publicly accessible areas on private land, private open space and communal open space.

# Table 4: Wind effects requirements

Wind condition	Requirement

Comfortable wind conditions

Unsafe wind conditions

Hourly mean wind speed or gust equivalent mean speed (3 second gust wind speed divided by 1.85), from all wind directions combined with probability of exceedance less than 20% of the time, equal to or less than:

- metres per second for sitting areas,

- metres per second for standing areas,

- metres per second for walking areas

Annual maximum 3 second gust wind speed exceeding 20 metres per second with a probability of exceedance of 0.1% considering at least 16 wind directions.

Access, parking and services

Ensure above ground car parking is sleeved with active uses along street frontages.

Minimise or consolidate vehicular access points and carefully design ramps into basements to support increased amenity and safety for pedestrians and bicycles.

Where possible,  avoid vehicular access from Church Street and Pakington Street..

Ensure car parking frontages to the public realm are visually interesting through materiality and detailing and allow for adequate ventilation.

Avoid solid roller shutter doors or security grills in Church Street and Pakington Street

Design off-street car parking facilities to have flexible electric vehicle charging spaces. Electric charging stations should be equipped with at least 50kw charging facilities.

Encourage shared car parking facilities according to peak car parking demand times.

Services, loading and waste areas should not be located on main street and public spaces. Where possible, they should be located within basements or on upper levels.

Integrate plant, equipment and services into the built form design. Where possible, avoid locating services on Pakington Street or grouping them together to create long inactive facades.

Site coverage

Development should not exceed the preferred maximum site coverage specified in Map 3 to this schedule.

Exemption from notice and review

An application or construct a building or construct or carry out works which accords with the height, setback and interface requirements of this clause is exempt from the notice requirements of section 52(1)(a), (b) and (d), the decision requirements of section 64(1), (2) and (3) and the

review rights of section 82(1) of the Act. This exemption does not apply to land within 30 metres of land (not a road) which is in a residential zone.

# 3.0

--/--/----

Proposed C433ggee

# Subdivision

The subdivision of land should not prevent the development of land in accordance with the objectives of this Schedule.

Exemption from notice and review

An application to subdivide land is exempt from the notice requirements of section 52(1)(a), (b) and (d), the decision requirements of section 64(1), (2) and (3) and the review rights of section 82(1) of the Act.

# 4.0

--/--/----

Proposed C433ggee

# Signs

None specified.

# 5.0

--/--/----

Proposed C433ggee

# Application requirements

The following application requirements apply to an application for a permit under Clause 43.02, in addition to those specified elsewhere in the scheme, and must accompany an application, as appropriate, to the satisfaction of the responsible authority:

An Urban Context Report and Design Response demonstrating how the proposal responds to the design objectives and buildings and works requirement of this schedule, responds to any adjacent residential properties, and implements recommendations from other technical reports.

Plans, elevations, and section drawings (with finished floor levels), including for any car parking at or above ground level.

Where car parking is proposed at ground or upper levels a statement by a suitably qualified engineer that demonstrates the capacity for adaptation to alternative uses.

Streetscape elevations showing the existing streetscape, and how the development sits within the streetscape and a three-dimensional perspective which shows the development in the context of adjacent development in the street.

A Wind Report prepared by a suitably qualified person for buildings exceeding a height of 16 metres) which addresses appropriate mitigation measures to achieve

safe and comfortable wind conditions on and nearby the site, without relying on street trees or excessive screening elements.

An Environmental Management Plan prepared by a suitably qualified person that demonstrates how the development provides for environmentally sustainable design measures.

A Traffic Impact Assessment Report prepared by a suitably qualified traffic engineer that assesses and minimises the impacts of traffic and parking within the precinct and promotes sustainable transport modes.

A Landscape Plan detailing proposed hard and soft landscape elements, plant schedule, plant container details and maintenance and irrigation systems.

Shadow diagrams to show existing and proposed shadows at hourly intervals to demonstrate compliance with the solar access requirements contained in this schedule.

Any application for development of land for a dwelling should provide an Affordable Housing Delivery Strategy to the satisfaction of the responsible authority which sets out the location and type (housing

type/density/size) of the affordable housing to be delivered, the method of implementation, and proposed staging which ensures affordable housing is provided in a timely manner as development occurs.

Any application for subdivision or development of land for Accommodation, Education centre (other than Tertiary institution and Employment training centre) or Hospital, must be accompanied by an acoustic assessment report prepared by a qualified acoustic engineer or

other suitably skilled person to the satisfaction of the responsible authority which:

–	Applies the following noise objectives:

Not greater than 35 dB LAeq,8h when measured within a sleeping area between 10pm and 6am.

Not greater than 40 dB LAeq,16h when measured within a living area between 6am and 10pm.

For areas other than sleeping and living areas, not greater than the median value of the range of recommended designed sound levels of Australian Standard AS/NZ 2107:2016 (Acoustics– Recommended design sound level and reverberation times for building interiors).

Train airborne noise received at new residential or other noise sensitive uses is attenuated to achieve a noise level of 55 dBA, Lmax in bedrooms at night and a noise level of 60 dBA, Lmax in living areas. These noise levels are to be measured at the expected occupancy position(s) in the space relevant to the noise of interest with doors and windows closed. The preferred positions are at least 1 metre from the walls or other major reflecting surface, 1.2 metres to 1.5 metres above the floor and about

1.5 metres from windows.

The measurements should be undertaken using a ‘fast’ meter time weighting and must be achieved for 95% of train pass- bys (i.e. 5%, 1 in 20 trains may exceed).

–	Noise levels should be assessed:

Considering the cumulative noise from all sources impacting on the proposal including road traffic, railway, industry and commercial noise, as well as other potential noise sources;

Industrial noise received at new residential or other noise sensitive uses achieves internal noise levels assessed in accordance with the Noise Protocol (EPA Publication 1826.4) with the implementation of an indoor adjustment of 20 dB, while allowing for operable windows. These noise levels are to be measured internally at the expected occupancy

position(s) in the space relevant to the noise of interest with doors and windows closed. The preferred positions are at least 1 metre from the walls or other major reflecting surface, 1.2 metres to 1.5 metres above the floor and about 1.5 metres from windows;

Operation of the rail sidings yard with respect to EPA Pub. 1826.4 Noise Protocol, where any new proposed sensitive uses constitutes the Agent of Change, and as such measures must be undertaken at sensitive uses to maintain EPA Pub. 1826.4 conformance of rail sidings yard; and

In unfurnished rooms with a finished floor and the windows closed and be based on average external noise levels measured as part of a noise level assessment.

- Addresses noise compatible design for buildings, with siting, orientation, and internal layout, to be considered prior to setting building envelope performance requirements.

- Addresses potential noise character (such as tonality, impulsiveness or intermittency)

wherever relevant, including through the application of adjustments to the internal noise levels that are determined using the procedures to adjust industry noise levels of the Noise Protocol.

# 6.0

--/--/----

Proposed C433ggee

# Decision guidelines

The following decision guidelines apply to an application for a permit under Clause 43.02, in addition to those specified in Clause 43.02 and elsewhere in the scheme which must be considered, as appropriate, by the Responsible Authority:

Whether the development employs an appropriate massing strategy that reduces visual bulk. Whether the development retains solar access to the locations specified in Table 3 to this schedule.

Whether the development adequately manages visual and internal amenity nd appropriately transitions to adjoining public open spaces, sensitive residential areas, heritage places or sites.

Whether the proposed design treatment and material selection is sympathetic to adjoining heritage place or site and transitions appropriately from the Heritage Core.

Whether the application includes an Affordable Housing Delivery Strategy to the satisfaction of the responsible authority.

Whether the development incorporates acoustic treatments to limit the impacts from noise from all current and potential noise generating sources such as railway operations, traffic and commercial activities.

Whether development contributes to an active and permeable streetscape, incorporates weather protection elements, and makes a positive contribution to the public realm.

Whether the development avoids blank walls, alcoves and recesses that provide hiding places or collect dirt and litter.

Whether the development achieves design excellence by providing high quality innovative architecture, landscape and urban design and provides community benefit.

Whether the development responds to the Rail Sidings Yard site in a manner appropriate for its current transport use.

Whether the development achieves comfortable wind conditions.

Whether the development is consistent with the Pakington North Urban Design Framework (City of Greater Geelong, May 2024).

Variations to preferred requirements

Where an application proposes to exceed, or vary a preferred requirement under a discretionary control contained within this schedule consider:

Whether the design objectives have been met.

Whether the development exceeds the minimum 5 star Greenstar rating for Environmentally Sustainable Design (ESD).

Whether the development results in, or substantially facilitates, the delivery of appropriately secured community benefits including:

- Incorporation of social or affordable housing.

- Upgrades or delivery of new local infrastructure including public spaces to meet the needs of the community and provide spaces for residents to linger and enjoy.

- Provision of pedestrian links or public open space in excess of any minimum requirement in this Scheme.

Whether development results in  adverse offsite impacts such as visual bulk, overlooking and overshadowing to adjoining residential properties and the public realm.

Whether the proposal presents, or substantially facilitates an improved architectural and urban design outcome.

Map 1 to Schedule 57 to Clause 43.02

[Editing Note: Delete information re. Railway Sidings site]

[Editing Note: Remove indicative open space icon from SE corner of Pakington Strand site]

Map 2 to Schedule 57 to Clause 43.02

[Editing Note: Delete information re. Railway Sidings site]

Map 3 to Schedule 57 to Clause 43.02

[Editing Note: Delete information re. Railway Sidings site]

[Editing Note: 60% site coverage to be changed to 70%)